AortaDiff: A Unified Multitask Diffusion Framework for Contrast-Free AAA Imaging

目 录

# AortaDiff: A Unified Multitask Diffusion Framework for Contrast-Free AAA Imaging

作者： Yuxuan Ou, Ning Bi, Jiazheng Pan, Jiancheng Yang, Boliang Yu, Usama Zidan, Regent Lee, Vicente Grau

## 摘要

While contrast-enhanced CT (CECT) is standard for assessing abdominal aortic aneurysms (AAA) , the required iodinated contrast agents pose significant risks, including patient allergies and environmental harm. To reduce contrast agent use, methods have focused on generating synthetic CECT from non-contrast CT (NCCT) scans. However, most adopt a multi-stage pipeline that first generates images and then performs segmentation, which leads to error accumulation and fails to leverage shared information. To address this, we propose a unified framework that generates synthetic CECT images from NCCT scans while simultaneously segmenting the aortic lumen and thrombus. Our approach integrates conditional diffusion models (CDM) with multi-task learning, enabling end-to-end joint optimization of image synthesis and anatomical segmentation.  Unlike previous multitask diffusion models, our approach requires no initial predictions (e.g., a coarse segmentation mask), shares both encoder and decoder parameters across tasks, and employs a semi-supervised training strategy to learn from scans with missing segmentation labels, a common constraint in clinical data.  Evaluated on a cohort of 264 patients, our method consistently outperformed state-of-the-art single-task and multi-stage models. For image synthesis, it achieved a PSNR of 25.61 dB, compared to 23.80 dB from a single-task CDM. For segmentation, it improved the lumen Dice score to 0.89 from 0.87 and the challenging thrombus Dice score to 0.53 from 0.48 (nnU-Net). These segmentation enhancements led to more accurate clinical measurements, reducing the lumen diameter MAE to 4.19 mm from 5.78 mm and the thrombus area error to 33.85% from 41.45%.

## 正文

This WACV paper is the Open Access version, provided by the Computer Vision Foundation.

Except for this watermark, it is identical to the accepted version;

the final published version of the proceedings is available on IEEE Xplore.

AortaDiff: A Uniﬁed Multitask Diffusion Framework for Contrast-Free AAA

Imaging

Yuxuan Ou1, Ning Bi1, Jiazheng Pan2, Jiancheng Yang3, Boliang Yu1, Usama Zidan1,

Regent Lee1, Vicente Grau1,

1University of Oxford

2Technical University of Munich

3Aalto University

yuxuan.ou@trinity.ox.ac.uk

Abstract                          1. Introduction

While contrast-enhanced CT (CECT) is standard for as-      Clinical assessment of vascular conditions like abdominal

sessing abdominal aortic aneurysms (AAA) , the required      aortic aneurysms (AAA) traditionally relies on contrast-

iodinated contrast agents pose signiﬁcant risks, including     enhanced CT (CECT), where an intravenous iodinated con-

patient allergies and environmental harm. To reduce con-       trast agent illuminates the aortic lumen to reveal abnormal-

trast agent use, methods have focused on generating syn-       ities such as thrombus [15, 22]. However, this procedure

thetic CECT from non-contrast CT (NCCT) scans. How-     poses risks to patients with renal insufﬁciency or contrast

ever, most adopt a multi-stage pipeline that ﬁrst generates      allergies, involves uncomfortable needle insertions, and

images and then performs segmentation, which leads to      contributes to environmental iodine waste [7, 10, 21, 27].

error accumulation and fails to leverage shared informa-     This has created a signiﬁcant clinical need for an advanced

tion. To address this, we propose a uniﬁed framework that     method capable of performing AAA assessment solely on

generates synthetic CECT images from NCCT scans while     non-contrast CT (NCCT) scans. Such an approach is par-

simultaneously segmenting the aortic lumen and throm-      ticularly challenging because thrombus and blood are nearly

bus. Our approach integrates conditional diffusion models      indistinguishable in NCCT, as shown in Figure 1 (b). Re-

(CDM) with multi-task learning, enabling end-to-end joint     cent advances in deep learning offer a promising pathway

optimization of image synthesis and anatomical segmenta-      to overcome this diagnostic challenge. An effective AI so-

tion.  Unlike previous multitask diffusion models, our ap-      lution must therefore perform two interrelated tasks:  (1)

proach requires no initial predictions (e.g., a coarse seg-     generate a realistic synthetic CECT to aid visual interpre-

mentation mask), shares both encoder and decoder param-      tation by radiologists, and (2) accurately segment the aor-

eters across tasks, and employs a semi-supervised train-       tic lumen, allowing thrombus to be identiﬁed as the non-

ing strategy to learn from scans with missing segmentation     enhancing regions outside this segmented area. This dual-

labels, a common constraint in clinical data.  Evaluated     output approach provides radiologists with both a qualita-

on a cohort of 264 patients, our method consistently out-      tive view of the anatomy from the synthetic image and im-

performed state-of-the-art single-task and multi-stage mod-     mediate quantitative measurements of the abnormality from

els. For image synthesis, it achieved a PSNR of 25.61 dB,      the segmentation mask, as illustrated in Figure 1 (a).

compared to 23.80 dB from a single-task CDM. For seg-       Recent AI advancements have primarily focused on gen-

mentation, it improved the lumen Dice score to 0.89 from      erating synthetic CECTs from NCCT scans, exploring ar-

0.87 and the challenging thrombus Dice score to 0.53 from      chitectures from early CNNs [19] to GANs [2, 4, 26] and

0.48 (nnU-Net). These segmentation enhancements led to     Transformers [17]. However, these works are constrained

more accurate clinical measurements, reducing the lumen     by a fundamental limitation: they frame the problem as a

diameter MAE to 4.19 mm from 5.78 mm and the thrombus      single-task, image-to-image translation. This overlooks the

area error to 33.85% from 41.45%. Code is at https:      clinical reality that a visually plausible image is not the end

//github.com/yuxuanou623/AortaDiff.             goal, but an intermediate step towards quantitative analysis.

Consequently, when segmentation is required, it is relegated

8242                                                                                                                                                                                                                    Thrombus                                                                                                     Review:

CECT                                                                                                 and    No IV Required        Single CT Aquisition     Non-ContrastInput: CT    MultitaskAortaDiffAlgorithm     SyntheticRadiologistSegmentation

Lumen

Aorta

AI-Enhanced Patient Journey for AAA Diagnosis                                           Non-Contrast CT                         Contrast-Enhanced CT     Annotated

(Lumen not visible)                       (Lumen enhanced)        Masks

(a)                                                                                           (b)

Accumulated Error

Consistency preserved

Syn.                                                    GT                                                                                                                                            Pred. GT Mask

Stage                                                                                        Stage                                 1                            Generation                                                                                 Error                                                                                        2                            Segmentation                                                                                                                                                      Error                                                                          Accuracy improved                         Modality                                     Translation                                                            CECT                                                                                           Segmentation

NCCT                                 Synthetic CECT                                      Mask Prediction

Multistage Methods: Error Accumulation Over Stages

Generation Error

Image-mask

Inconsistency                                                                       Joint NCCT to CECT

Model                                  1                                                                     NCCT       & Segmentation Model              Synthetic CECT & Segmentation

NCCT                                        to CECT

NCCT                                                             Segmentation Error                  CECT

Model 2

NCCT Segmentation                                                                                                                              AortaDiff Models:

Single-Task Methods: Output Inconsistent                                                          Simultaneously generate synthetic CECT and perform segmentation

(c)

Figure 1. Uniﬁed Multitask Framework for Contrast-Free AAA Assessment. This ﬁgure illustrates the clinical motivation, CT imaging

concepts, and advantages of our proposed multitask diffusion model (AortaDiff) over traditional approaches. (a) The proposed AI-enhanced

clinical workﬂow enables diagnosis from a NCCT scan, eliminating the need for intravenous (IV) contrast agents. (b) A visual comparison

highlights the diagnostic challenge: the aortic lumen is obscured in the NCCT but is clearly delineated in the CECT. (c) Our uniﬁed

multitask model is contrasted with prior methods. Multistage pipelines risk accumulating synthesis errors into the ﬁnal segmentation.

Single-task approaches can produce segmentations that are inconsistent with the synthesized image. In contrast, our end-to-end model

jointly generates a high-ﬁdelity synthetic CECT and an accurate, consistent segmentation mask.

to a separate, downstream model [2, 4]. This multi-stage     image translation from NCCT to CECT and lumen segmen-

pipeline is inherently ﬂawed, as it is susceptible to error ac-      tation. The core of our approach is a novel denoising U-

cumulation, where synthesis artifacts degrade segmentation     Net architecture featuring a shared encoder-decoder back-

accuracy, and prevents knowledge sharing between the two     bone that ends in two different prediction heads: one for the

highly relevant tasks.                                       denoised image and one for the segmentation mask. This

single-backbone design allows the network to learn a rich,  To address the shortcomings of multi-stage methods,

shared latent representation that includes both texture ofsome pioneering works have explored uniﬁed, multi-task

contrast enhancement and precise anatomical informationframeworks. For instance, Hu et al.  [8] and UCAS [34]

about lumen thrombus boundary. What’s more, our modelproposed models that jointly perform CECT synthesis and

advances beyond prior multitask diffusion methods in twosegmentation. However, these early multi-task attempts are

ways.  First, it operates directly on the raw NCCT input,predominantly built upon GAN-based architectures. While

eliminating the need for any coarse initial predictions (e.g.,innovative in their task formulation, they inherit the well-

a pre-computed coarse segmentation mask) during bothknown challenges of GANs:  they often produce overly

training and inference. Second, to address the speciﬁc chal-smooth outputs that lack the sharp anatomical boundaries

lenge of scarce lumen segmentation labels, we introducecritical for clinical diagnosis and can suffer from training

a semi-supervised training strategy. This strategy incorpo-instability or mode collapse [1, 16]. Therefore, a signiﬁcant

rates a loss function that dynamically changes between su-gap remains for a framework that not only adopts a uni-

pervised objectives on fully labeled data with an unsuper-ﬁed multi-task design but also leverages a more powerful

vised reconstruction objective on image-only pairs.  Thisgenerative model to ensure both high-ﬁdelity synthesis and

allows our model to effectively leverage the entire dataset,precise segmentation.

even when only a subset of scans have lumen masks, en-

To address these limitations, we propose a uniﬁed multi-     hancing its generalization capabilities under limited super-

task diffusion framework, AortaDiff, that jointly performs

8243vision and aligning with the practical constraints of clinical      that branches into task-speciﬁc decoders, conditioning the

data acquisition.                                                 diffusion process on their initial outputs.  Other works,

We conducted a comprehensive evaluation of our frame-      like Shadow Diffusion [6], require a coarse initial mask

work on the Oxford Abdominal Aortic Aneurysm (Ox-      to guide the joint reﬁnement process.  Focusing on efﬁ-

AAA) dataset [25]. Our uniﬁed model was benchmarked      ciency, MTU [3] adapts pre-trained models by replacing

against state-of-the-art baselines and traditional multi-stage     network layers with smaller, task-speciﬁc feed-forward net-

pipelines. The results demonstrate a consistent improve-    work and proposing a routing mechanism.  While these

ment across all tasks. For the challenging thrombus seg-     general-purpose frameworks are foundational, they are not

mentation, our model increases the Dice score from 0.48      tailored for the unique constraints of medical imaging, such

(nnU-Net) to 0.53, and for lumen segmentation, from 0.87     as the need for high anatomical precision and handling of

to 0.89.  In the synthesis task, our framework achieves a     scarce clinical data.

PSNR of 25.61 dB, markedly outperforming the single-task        Within the medical domain, a few works have begun to

CDM’s 23.80 dB. Qualitatively, our jointly generated out-     adapt these concepts. DiffAtlas [31], for example, applied

puts exhibit sharper anatomical details and superior seg-     a joint image-mask generation model to medical images but

mentation consistency. We also validate our method on two    was limited by its requirement for full supervision. While

AAA clinical measurements and found that it yields more       all these works have signiﬁcantly advanced multi-task dif-

accurate morphological predictions for both lumen diame-      fusion, a critical gap remains for a framework that is simul-

ter and thrombus area, underscoring its potential for reliable     taneously (1) architecturally uniﬁed and specialized for a

clinical application.                                        medical task, (2) initialization-free and label-efﬁcient via a

In summary, the contributions of this work are threefold:     semi-supervised scheme, and (3) validated on a high-stakes

• We propose a novel diffusion-based multi-task framework      clinical application. Our work is designed to ﬁll this speciﬁc

designed to move beyond image synthesis towards clini-      void.

cally actionable analysis. Based on the insight that a syn-

thetic CECT is an intermediate step, our model jointly

performs synthetic CECT generation and lumen segmen-     3. Proposed Method: AortaDiff

tation. This approach enables a safer and more sustain-                                        We propose AortaDiff, a novel multi-task diffusion-based

able workﬂow for AAA imaging by reducing the need for                                                      framework that jointly generates synthetic CECT images

contrast agent injection.                                                      and performs lumen segmentation directly from NCCT

• To address the real-world challenge of missing lumen seg-                                                                 scans. The method focuses explicitly on the aortic region,

mentation annotations, we introduce a semi-supervised                                                            simultaneously synthesizing the CECT image and its cor-

strategy  that enables effective learning from datasets                                                          responding lumen mask while preserving the surrounding

where only a subset of image pairs contains lumen seg-                                                         background.  Furthermore, we employ a semi-supervised

mentation masks. This strategy improves both generation                                                                   training strategy that effectively learns both tasks by lever-

and segmentation performance beyond fully supervised                                                          aging a combination of data with and without lumen mask

training, leveraging cross-task consistency and multi-task                                                                annotations.

representation learning.

• Experiments on the OxAAA dataset demonstrate that our

model outperforms all single-task baselines and multi-                     Example of a Misaligned Image Pair

stage pipelines on both generation and segmentation

tasks. Our method is also more label-efﬁcient than prior

multi-task diffusion approaches, making  it suitable for

scalable deployment in resource-constrained healthcare

environments.

Related Work: Multi-task Diffusion Models                          Non-Contrast CT       Contrast-Enhanced CT

Extending diffusion models for multi-task learning is an      Figure 2.  Misalignments in Paired Scans from the OxAAA

active area of research, with many foundational strategies      Dataset.  An example of spatial discrepancies between corre-

ﬁrst developed in general computer vision.  For instance,     sponding NCCT and CECT slices from the OxAAA dataset. This

illustrates the inherent difﬁculty of achieving perfect pixel-wiseCoDi [20] enables ’any-to-any’ generation by aligning sep-

alignment in practice, even after rigid registration. Non rigid reg-arate, pre-trained diffusers in a shared latent space via cross-                                                                               istration is not used and the results are in the supplementary mate-

attention.  TaskDiffusion [28] introduces a joint denois-                                                                                        rials.

ing process with a cross-task decoder to unify dense pre-

dictions, while DiffusionMTL [29] uses a shared encoder

82443.1. Aorta-Focused Generation with Background     U-Net decoder. The ﬁrst head outputs the predicted im-

Preservation                                      age noise, as is standard in DDPMs. For the second head,

to encourage smoother and more precise lumen boundaryA primary challenge in paired NCCT-CECT datasets is spa-

prediction, we represent the lumen mask as a Signed Dis-tial misalignment due to factors like respiratory motion,

tance Function (SDF). This head therefore outputs a pre-even after standard image registration [1]. An example is

dicted SDF, ˆs0. This uniﬁed design forces the shared back-shown in Figure 2.  To prevent the model from learning

bone to learn a rich latent representation beneﬁcial for bothspurious translations of surrounding tissues, such as the air-

tasks. To balance these distinct objectives, we employ theways, we constrain the generative process to focus exclu-

principled uncertainty-weighting strategy from Kendall etsively on the aortic region, shown in Figure 1 (b), while

al. [11].  This approach introduces two trainable parame-preserving the original NCCT background.

ters, σ1 and σ2, to automatically learn the task weightings

by optimizing the joint objective:

Aorta-Focused Training Strategy  Inspired by [30], our

training objective is localized to a bounding box tightly en-                   1            1                                                                  σ1, σ2) =                              σ1σ2  ,closing the aorta, which is determined from the ground truth    L(W,         2σ21 Lseg(W)+ 2σ22 Lrecon(W)+logaorta segmentation masks. Within this bounding box, we                                                   !   (2)"

replace the standard pixel-wise MSE loss with a perceptual     where W are the model parameters, Lrecon is the perceptual                                                        on the image                                                                                  patches,                                                                                                                          is the L1                                                                                                                  loss betweenloss         [32]. This                    loss is less                                 sensitive                                          to the minor                                                       pixel      loss                                                                             and Lseg   Lp                                                        =                                                                the predicted                                                                  and ground                                                                                                 truth                                                                             SDFs:shifts      inherent               in misaligned                            data                            and encourages                                                 the model                                                                                                       Lseg     kˆs0 −s0k1.

to reconstruct the aorta texture and morphology faithfully.     This formulation provides a robust and well-founded ap-

The reconstruction loss is therefore calculated as:              proach to jointly optimizing both the image synthesis and

geometric segmentation tasks.

Lrecon = Lp(x0[B], ˆx0[B]),              (1)     3.3. Semi-Supervised Training Strategy

where x0 and ˆx0 are the ground truth and predicted CECT    To effectively leverage the entire OxAAA dataset, which

images, respectively, and B denotes the patch deﬁned by the     contains 65 cases with both NCCT-CECT pair and lumen

aorta-enclosing bounding box.                          mask and 199 cases without lumen mask, we introduce a

semi-supervised learning framework. Rather than incorpo-

Inference and Fusion Strategy  During inference, we      rating complex SSL frameworks, we adopt a direct semi-

ﬁrst obtain an aorta segmentation mask on NCCT scans us-     supervised strategy, leveraging the inherent representation

ing a pre-trained nnU-Net.  It is trained exclusively on the      learning strengths of the diffusion model. Our strategy is

training split with strict patient-level separation, using only     designed to integrate both data types into the training pro-

NCCT scans and aorta labels. The model is frozen before     cess seamlessly, addressing the common challenge of label

any test evaluation and achieves a Dice score of 0.93 on      scarcity in clinical applications. The training proceeds in

the validation set. To generate the ﬁnal image while pre-     two phases.  First, we perform a pre-training phase where

serving the background, we adapt the inpainting strategy      the model is warmed up exclusively on the 65 fully-labeled

from RePaint [13]. At each denoising timestep t, the known     samples for one epoch. This ensures the segmentation head

background region is directly replaced by the corresponding      receives a stable initial gradient signal. Following this, we

pixels from the input NCCT scan after applying the forward     begin the mixed-training phase, where each training batch

diffusion process noise scheduled for that step.  Concur-       is constructed by sampling from both the labeled and unla-

rently, the foreground aorta region is ﬁlled with our model’s     beled pools at a 1:1 ratio. For samples with ground truth

denoised prediction. This composition ensures a seamless     segmentation, the model is optimized using the combined

fusion of the synthesized aorta into its original anatomical      loss mentioned in the previous subsection.  For samples

context. To further enhance boundary coherence, we em-     lacking a segmentation mask, the segmentation loss is omit-

ploy a resampling scheme [13], where the denoising pro-      ted, and the network is updated solely by the reconstruction

cess is interleaved with several noising steps, allowing the      loss. This allows the shared feature backbone of our diffu-

model to better harmonize the foreground and background.      sion model to learn robust anatomical representations from

the entire corpus of 264 cases. The overall semi-supervised

3.2. A Uniﬁed Architecture for Multitask Denoising      training strategy is illustrated in Figure 3 (a).

To enable multi-task learning, we adapt the standard de-

noising U-Net architecture to simultaneously predict both     AortaDiff Workﬂow  During  training,  the  AortaDiff

the denoised image and its corresponding lumen segmenta-     model follows the conditional diffusion model framework:

tion. We implement this by incorporating two parallel pre-      the clean CECT image x0 is progressively noised to obtain

diction heads that branch from the ﬁnal layer of the shared     a corrupted version xt, while the corresponding NCCT scan

8245                                                                                            Reconstruction Loss

Reconstruction Loss

Real CECT                  Real CECT           Noised CECT                                  Denoised CECT         Gaussian Noise                                    Predicted CECT

c       AortaDiff                                           c       AortaDiff

or

Lumen Mask

Lumen Mask

No Lumen Mask           Lumen Mask            Real NCCT                                           Prediction              Real NCCT                                          Prediction

Semi                          Full

Supervision                 Supervision                  Input                                          Output                    Input                                          Output

Labels                      Labels

Segmentation Loss

Segmentation loss skipped: no mask available

(a)   Training                                                                     (b)    Inference

Figure 3. Overview of the AortaDiff Training and Inference Workﬂow. The model is trained using a semi-supervised strategy. For all

data, the model is updated with a reconstruction loss. An additional segmentation loss is applied only when a lumen mask is available.

In inference, starting with an NCCT scan and a pure Gaussian noise image, the model performs iterative denoising to jointly generate a

synthetic CECT and its corresponding lumen mask.

serves as the conditional input c0 throughout the reverse de-     stage baselines that ﬁrst generate synthetic CECT and then

noising process. At each timestep t, the model receives the     segment the result. Both qualitative and quantitative results

concatenated pair (xt, c0) along with the timestep embed-     demonstrate the effectiveness of our proposed approach.

ding, and predicts both the noise ˆ✏t for image reconstruction

ˆx0 and lumen segmentation ˆs0. These two outputs are su-     4.1. OxAAA Dataset

pervised based on label availability: when both CECT and                                                  The OxAAA dataset consists of 264 patient cases, each in-

lumen annotations are available, the model is trained using a                                                             cluding spatially registered NCCT and CECT scans [25].

multi-task objective. When the lumen mask is unavailable,                                                         Aorta segmentation labels are available for all cases, while

only the reconstruction loss is applied. During inference,                                                          only 65 cases additionally include expert-annotated lumen

the model takes only the NCCT image as input.  Starting                                                         masks. We use 45 of the labeled cases for training and val-

from Gaussian noise, it performs iterative denoising condi-                                                                     idation, and reserve 20 for testing. In our semi-supervised

tioned on the NCCT over the full diffusion trajectory. The                                                                       setting, the remaining 199 cases without lumen annotations

ﬁnal outputs are the synthesized CECT image and the cor-                                                                are used during training.

responding lumen mask. The inference workﬂow is shown

in Figure 3 (b).                                                4.2. Dataset Preprocessing

All CT scans used in our study are 3D volumes from which4. Experiments                                             we extract 2D slices along the axial plane. To reduce re-

We  train and evaluate the proposed multi-tasking  dif-     dundancy from high inter-slice similarity, we retain one

fusion  models,  AortaDiff,  on  the Abdominal  Aortic     out of every three axial slices per volume, thereby balanc-

Aneurysm dataset released by the Oxford Abdominal Aor-     ing dataset size and information content. To ensure spatial

tic Aneurysm (OxAAA) Study [2, 25]. We assess two vari-     alignment between NCCT and CECT scans, we retain only

ants of our model: AortaDiff-F, trained with full supervi-      slice pairs with an aorta mask dice score above 0.9.

sion on fully-labeled data, and AortaDiff-P, trained with      As lumen annotations are available only on the CECT

incomplete labels using our semi-supervised strategy on the      scans, we transfer the corresponding lumen mask to the

entire dataset.                                                 aligned NCCT slice. This process yields a total of 4,370

Performance is assessed on two fronts: synthetic CECT     paired triplets, each consisting of an NCCT slice, its aligned

image generation and segmentation of the aortic lumen and    CECT counterpart, and a corresponding lumen segmenta-

thrombus. For each task, we compare our method against      tion mask.

state-of-the-art single-task models trained with full supervi-      From these, we randomly select 2,823 triplets for train-

sion. In the segmentation task, we also evaluate the multi-     ing and 1,547 for testing. To avoid data leakage, all slices

8246from the same patient are assigned exclusively to either      that incorporating perceptual loss improves performance

the training or testing split. In addition, we include 7,366     across all models.

NCCT-CECT slice pairs without lumen annotations, which    Lumen Segmentation. For the lumen segmentation task,

are used during semi-supervised training.                we compare our method with nnU-Net [9], a state-of-the-

Hounsﬁeld Units are clipped to the range [−1000, 1000],      art medical image segmentation framework that achieves

followed by min-max normalization to [−1, 1] on a per-slice     strong performance.  Additionally, we include MedSAMbasis before input to the model.                                   [14], a pretrained vision foundation model for medical im-

age segmentation. For MedSAM, we use the publicly re-

4.3. Experimental Settings                                 leased pretrained weights without any task-speciﬁc ﬁne-

tuning.

We adopt the AdamW optimizer with a learning rate of 1 ⇥

10−4 and no weight decay [12]. The model is trained with a     4.6. Experiment Results

batch size of 16 and an input image matrix size of 256⇥256.Training is conducted for a total of 106 iterations.                4.6.1. Quantitative Evaluation of CECT Synthesis

For the DDPM, we use a linear noise schedule with 1,000     Table 1 presents quantitative results for CECT synthesis

diffusion timesteps.                                                 task, evaluated within the local aortic region.  We ob-

serve that our proposed aorta-restricted generation strategy

4.4. Evaluation Metrics                          (CDM-A) outperforms both CDM-W, CycleGAN and U-

Net across all metrics. CDM-W and CycleGAN underper-For the image generation task, we evaluate the quality of the

form U-Net. This performance gap highlights the sensitiv-synthesized CECT slices using Peak Signal-to-Noise Ratio

ity of diffusion and GAN models to irrelevant regions in the(PSNR), Structural Similarity Index Measure (SSIM) [23],

conditioning input, such as those introduced by imperfectand Learned Perceptual Image Patch Similarity (LPIPS)

alignment between NCCT and CECT scans. These ﬁnd-[32].  Given the anatomical misalignment between the

ings underscore the necessity of region-of-interest-focusedNCCT and CECT, particularly outside the aorta region, we

generation to ensure stable performance in anatomically lo-focus our evaluation on the region of interest — the aorta.

calized tasks.To reduce the inﬂuence of background artifacts and regis-

Our proposed multi-task diffusion models (AortaDiff-Ftration errors, we crop each slice to the smallest bounding

and AortaDiff-P) achieve the best and second-best perfor-box that encloses the aorta and compute the metrics within

mance across all evaluation metrics.  This consistent im-this cropped region.

provement demonstrates that incorporating the segmenta-

4.5. Baselines                                                   tion task enhances the model’s ability to capture meaning-

ful anatomical structure, thereby improving image synthesis

Synthetic CECT Generation. To evaluate the performance      quality. The results support the hypothesis that joint learn-

of our method on synthetic CECT generation, we consider                                                             ing enables the model to leverage semantic guidance from

three baseline architectures: a U-Net [18], a CycleGAN[33]                                                                the segmentation objective, leading to more accurate and

and a conditional diffusion model (CDM) [5]. The noise                                                            anatomically coherent CECT generation.

prediction network used in all diffusion models, including

our proposed method, shares the same U-Net backbone for a

Method    PSNR (dB)   SSIM    LPIPSfair architectural comparison. The CycleGAN experiments                              "        "         #

follow the default conﬁgurations provided in the ofﬁcial        Generation-Only Methods

codebase. For the CDM baseline, we evaluate two variants:       UNet            24.40       0.7785    0.0815

CDM-W (Whole Image):The entire NCCT slice is used as       CycleGAN       23.19       0.6535    0.1129

the conditioning input to generate the full CECT image.       CDM-W         23.80       0.7616    0.0859

CDM-A (Aorta-Restricted): Generation is restricted to the      CDM-A          25.25       0.8137    0.0750

aortic region by conditioning only within the bounding box         Multi-Task Methods (Ours)

of the aorta.                                                    AortaDiff-F      25.61       0.8385    0.0671

This comparison allows us to assess whether focusing        AortaDiff-P      25.48       0.8296    0.0606

generation on the anatomically relevant region yields better

results than generating the entire image. For the U-Net and      Table 1.  Quantitative Comparison on the NCCT-to-CECT

CycleGAN baselines, the conditioning input is the whole      Translation Task. All metrics are computed within the local re-

NCCT image.                                                   gion surrounding the aorta. The best and second-best values are

Due to misalignment between NCCT and CECT images,      highlighted in bold and underlined, respectively. Our multi-task

we replace the default MSE loss with a perceptual loss for      diffusion models outperform baselines in all metrics.

all baseline models. In our ablation study, we demonstrate

8247    Input NCCT and GT CECT         Generation Only Methods      Segmentation Only Methods  Multi-Task Methods

Real          Real         (Segmentation performed by another model: nnUNet)         (No CECT Generated)

NCCT      CECT         UNet     CDM-W     CDM-A        nnUNet    MedSAM     AortaDiff-F   AortaDiff-P

✖  ✖

1

Case

✖  ✖

L: 0.89 T: 0.18      L: 0.63 T: 0.13       L: 0.93 T: 0.29        L: 0.53 T: 0.12      L: 0.89 T: 0.23        L: 0.96 T: 0.41     L: 0.95 T: 0.38

✖  ✖

2

Case

✖  ✖

L: 0.78 T: 0.59     L: 0.80 T: 0.58      L: 0.74 T: 0.54        L: 0.82 T: 0.64       L: 0.74 T: 0.67       L: 0.79 T: 0.66     L: 0.91 T: 0.87

Figure 4. Qualitative Comparison of Synthetic CECT Generation and Lumen and Thrombus Segmentation. This ﬁgure illustrates

two representative cases comparing generation-only (two-stage pipeline), segmentation-only, and our proposed multi-task diffusion meth-

ods. For each case, the top row displays the full generated image, while the bottom row provides a zoomed-in view of the aortic region with

the corresponding segmentation result. In both cases, our multi-task methods achieve the most accurate segmentation with clearly deﬁned

boundaries and precise localization. Note that for generation-only methods, the segmentation is performed by nnU-Net on the generated

synthetic CECT. For segmentation-only methods, no synthetic CECT is produced; thus, the corresponding CT images are shown in gray.

Lumen     Thrombus           compute the thrombus mask by subtracting the predicted     Method                   Dice Score    Dice Score            lumen region from the aorta region. This subtraction-based                            "            "                                                         approach aligns with clinical deﬁnitions, where the throm-

Segmentation-Only Methods                                                         bus is the area enclosed by the aortic wall but outside the

MedSAM       0.7479        0.4356                                                                 contrast-ﬁlled lumen.

nnUNet         0.8718        0.4750

Table 2 presents Dice scores for both lumen and throm-

Generation-Only Methods (seg. by nnUNet)            bus segmentation. We compare three groups of models: (1)

UNet           0.8078        0.4604               segmentation-only baselines (MedSAM and nnU-Net), (2)

CycleGAN      0.6770        0.4907               generation-only methods, where nnU-Net is applied to syn-

CDM-W        0.7748        0.4339                 thetic CECT images, and (3) our proposed multi-task diffu-

CDM-A         0.8908        0.4914                sion models (AortaDiff-F and AortaDiff-P).

Multi-Task Methods (Ours)                          The  results reveal several key ﬁndings.    First,  for

AortaDiff-F     0.8887        0.5182               generation-only method, CDM-A outperforms both CDM-

AortaDiff-P     0.8933        0.5326            W, CycleGAN and U-Net in both tasks, conﬁrming that

ROI-constrained generation improves downstream segmen-

Table 2. Quantitative Comparison on the Lumen and Throm-      tation quality.  Second, AortaDiff-P achieves the high-

bus Segmentation Tasks. For generation-only methods, segmen-      est Dice scores across both tasks, 0.8933 for lumen and

tation is performed by applying nnU-Net to the generated synthetic     0.5326  for thrombus,  exceeding even  fully supervised

CECT images. Our semi-supervised multi-task diffusion models     segmentation-only methods.

achieve superior performance on both tasks.                                                     Thrombus segmentation remains more challenging than

lumen segmentation, as indicated by lower Dice scores.

This is likely due to the thrombus often appearing as thin

4.6.2. Quantitative Evaluation of Lumen and Thrombus      rings with fewer pixels. Nevertheless, the superior perfor-

Segmentation                                 mance of our AortaDiff models suggests that jointly op-

We evaluate lumen segmentation performance directly us-     timizing the generation and segmentation tasks allows the

ing each model’s predicted segmentation output. For throm-     model to learn more accurate anatomical representations of

bus segmentation, we ﬁrst obtain aorta masks at test time      the lumen and surrounding structures.

using the pretrained nnU-Net described above. We then        Importantly,  our  semi-supervised  setup  enables  the

8248model to beneﬁt from training data without lumen segmen-      Table 3. Lumen Diameter Regression on the Test Set. Best per

tation labels. Even in such cases, the translation task facili-     column in bold.

tates learning structural cues that improve segmentation per-

formance. This is particularly valuable in real-world clini-    Method   MAE (mm) # RMSE (mm) # Pearson r "

cal settings, where unlabeled imaging data are abundant, but      Baseline

high-quality annotations are expensive and labor-intensive                                                    nnU-Net         5.78           9.90          0.91

to obtain. By learning jointly from both labeled and unla-

Oursbeled data, our model demonstrates strong cross-task gen-

AortaDiff-F      4.70           7.05          0.93eralization.

AortaDiff-P      4.19           6.34          0.94

4.6.3. Qualitative Evaluation of CECT Synthesis and Lu-

men/Thrombus Segmentation                          Table 4. Thrombus Area Prediction Error vs. Ground Truth.

Figure 4 presents two challenging clinical cases to quali-      Best per column in bold.

tatively compare all baseline and proposed methods across

both the CECT synthesis and segmentation tasks.                     Method   Mean ± SD (%) #

In Case 1, the reference CECT reveals a large, circular                 Baseline

lumen with a thin but clearly visible mural thrombus en-               nnU-Net    41.45%  29.53%circling the vessel wall, corresponding to a circumferen-                     ±

Ourstial thrombus morphology. UNet and CDM-W fail to syn-

AortaDiff-F 36.89%  19.39%thesize the enhanced lumen correctly, exhibiting mottled                     ±                                                                       AortaDiff-P 33.85%  16.52%textures and anatomically implausible aortic appearances.                     ±

While CDM-A recovers a large circular lumen, it lacks con-

trast enhancement and fails to delineate the aortic boundary.

outperform nnU-Net, indicating strong potential for clinicalIn contrast, AortaDiff-F and AortaDiff-P produce sharper,

application.anatomically realistic lumen enhancement and better pre-

serve the aortic wall. On the segmentation task, AortaDiff

models accurately localize the lumen, achieving the highest     5. Conclusions

Dice scores among all methods. Although the thrombus in                                                              In this work, we introduced AortaDiff, a uniﬁed multi-task

this case is relatively thin and thus difﬁcult to delineate pre-                                                                 diffusion framework for generating synthetic CECTs and

cisely, both AortaDiff variants outperform all comparison                                                         segmenting the aortic lumen directly from NCCT scans.

methods in identifying its presence and extent.                                        We demonstrated that having a diffusion model perform

In Case 2, the thrombus extends along the posterolat-                                                               these tasks simultaneously enhances performance across

eral and lateral walls of the aorta, while the lumen appears                                                          both synthesis and segmentation. We also showed that an

as a bright, elliptical structure located in the inferior por-                                                              aorta-focused generation strategy with a background fu-

tion of the aortic cross-section. Only AortaDiff-P correctly                                                              sion technique can overcome signiﬁcant data misalignment

synthesizes the shape, contrast, and position of the lumen,                                                                     issues, while a direct, semi-supervised training scheme

along with a clear interface between lumen and thrombus.                                                             enables learning from annotation-scarce clinical datasets.

For segmentation, other methods incorrectly predict the lu-                                                         Experiments show that our multitasking approach outper-

men as a large, centrally located circle, leading to under-                                                      formed state-of-the-art single-task and multi-stage base-

segmentation of the thrombus. In contrast, AortaDiff mod-                                                                     lines on both synthesis and segmentation, leading to more

els yield accurate segmentation of both the inferolaterally                                                              accurate AAA clinical measurements. This research repre-

located lumen and the eccentric thrombus.                                                                 sents a meaningful step towards creating a “digital contrast”

These results demonstrate that our multi-task diffusion                                                                   for CT.

models not only generate anatomically consistent synthetic        Despite these contributions, limitations remain.  First,

CECT images but also deliver clinically meaningful seg-    DDPM-based models have longer inference time than Cy-

mentation results.                                  cleGAN or U-Net based models, and the use of RePaint

further increases inference time, posing challenges for clin-

4.7. Clinical Measurements for AAA                         ical deployment. Second, our method currently requires lu-

men masks during training; since the model learns to gener-

Aneurysm morphology is important for predicting AAA                                                                   ate the lumen, future research could investigate leveraging

progression [24, 25]. We evaluate two clinically relevant      latent representations for unsupervised lumen prediction.

metrics, lumen diameter and thrombus area, on a random     Third, uncertainty estimation is crucial for clinical trust,

sample of ten test cases. As shown in Tables 3 and 4, our     motivating future work on quantifying uncertainty in both

models produce accurate predictions on both metrics and     generation and segmentation.

8249References                                                      [15] Vinicio Napoli, Irene Bargellini, Savino G Sardella, Pasquale

Petruzzi, Roberto Cioni, Claudio Vignali, Mauro Ferrari, and

[1] Ghazal Azarfar, Seok-Bum Ko, Scott J Adams, and Paul S                                                                      Carlo Bartolozzi.  Abdominal aortic aneurysm:  contrast-

Babyn.  Applications of deep learning to reduce the need                                                                  enhanced us for missed endoleaks after endoluminal repair.

for iodinated contrast media for ct imaging: a systematic re-                                                                       Radiology, 233(1):217–225, 2004. 1

view. International journal of computer assisted radiology

[16] Emmanuel Oladokun, Yuxuan Ou, Anna Novikova, Daria    and surgery, 18(10):1903–1914, 2023. 2, 4                                                                        Kulikova, Sarina Thomas, Jurica ˇSprem, and Vicente Grau.

[2] Anirudh  Chandrashekar,  Ashok  Handa,  Pierfrancesco                                                         From transthoracic to transesophageal: Cross-modality gen-

Lapolla, Natesh Shivakumar, Raman Uberoi, Vicente Grau,                                                                              eration using lora diffusion.  In Simplifying Medical Ul-

and Regent Lee. A deep learning approach to visualize aortic                                                                          trasound, pages 120–129, Cham, 2026. Springer Nature

aneurysm morphology without the use of intravenous con-                                                                          Switzerland. 2

trast agents. Annals of Surgery, 277(2):e449–e459, 2023. 1,

[17] Nicolae-C˘at˘alin  Ristea,  Andreea-Iuliana Miron,  Olivian      2, 5

Savencu, Mariana-Iuliana Georgescu, Nicolae Verga, Fa-

[3] Ruchika Chavhan, Abhinav Mehrotra, Malcolm Chadwick,                                                                 had Shahbaz Khan, and Radu Tudor Ionescu.  Cytran: A

Alberto Gil Ramos, Luca Morreale, Mehdi Noroozi, and                                                                             cycle-consistent transformer with multi-level consistency for

Sourav Bhattacharya.   Upcycling text-to-image diffusion                                                                           non-contrast to contrast ct translation. Neurocomputing, 538:

models for multi-task capabilities, 2025. 3                                                                   126211, 2023. 1

[4] Jaehee Chun, Jee Suk Chang, Caleb Oh, InKyung Park,                                                                     [18] Olaf Ronneberger, Philipp Fischer, and Thomas Brox. U-net:

Min Seo Choi, Chae-Seon Hong, Hojin Kim, Gowoon Yang,                                                                       Convolutional networks for biomedical image segmentation,

Jin Young Moon, Seung Yeun Chung,  et  al.   Synthetic                                                                     2015. 6

contrast-enhanced computed tomography generation using a

[19] Gianmarco  Santini, Lorena M Zumbo,  Nicola Martini,     deep convolutional neural network for cardiac substructure

Gabriele Valvano, Andrea Leo, Andrea Ripoli, Francesco     delineation in breast cancer radiation therapy: a feasibility

Avogliero, Dante Chiappino, and Daniele Della Latta. Syn-      study. Radiation Oncology, 17(1):83, 2022. 1, 2

thetic contrast enhancement in cardiac ct with deep learning.

[5] Prafulla Dhariwal and Alex Nichol. Diffusion models beat                                                                      arXiv preprint arXiv:1807.01779, 2018. 1

gans on image synthesis, 2021. 6

[20] Zineng Tang, Ziyi Yang, Chenguang Zhu, Michael Zeng, and

[6] Lanqing Guo, Chong Wang, Wenhan Yang, Siyu Huang,                                                                Mohit Bansal. Any-to-any generation via composable diffu-

Yufei Wang, Hanspeter Pﬁster, and Bihan Wen. Shadowd-                                                                                 sion, 2023. 3

iffusion: When degradation prior meets diffusion model for

[21] Gail Ter Haar. Safety and bio-effects of ultrasound contrast    shadow removal, 2022. 3

agents. Medical & biological engineering & computing, 47

[7] Martina C Holder, Daniel A Lewis, and P Shane Win-                                                                        (8):893–900, 2009. 1

stead. Preventing the harm of a closer look: contrast-induced

[22] Yiying Wang, Abhirup Banerjee, Robin P. Choudhury, and     nephropathy in adults. Orthopedics, 35(4):298–303, 2012. 1

Vicente Grau.  Deepca: Deep learning-based 3d coronary

[8] Tao Hu, Masahiro Oda, Yuichiro Hayashi, Zhongyang Lu,

artery tree reconstruction from two 2d non-simultaneous x-

Kanako Kunishima Kumamaru, Toshiaki Akashi, Shigeki

ray angiography projections.  In 2025 IEEE/CVF Winter

Aoki, and Kensaku Mori. Aorta-aware gan for non-contrast

Conference on Applications of Computer Vision (WACV),

to artery contrasted ct translation and its application to ab-

pages 337–346, 2025. 1

dominal aortic aneurysm detection. International Journal of

[23] Zhou Wang, A.C. Bovik, H.R. Sheikh, and E.P. Simoncelli.    Computer Assisted Radiology and Surgery, pages 1–9, 2022.

Image quality assessment: from error visibility to structural    2

similarity. IEEE Transactions on Image Processing, 13(4): [9] Fabian Isensee, Paul F Jaeger, Simon A A Kohl, Jens Pe-

600–612, 2004. 6      tersen, and Klaus H Maier-Hein. nnU-Net: a self-conﬁguring

[24] Anders Wanhainen, Fabio Verzini, Isabelle Van Herzeele,    method for deep learning-based biomedical image segmen-

Eric Allaire, Matthew Bown, Tina Cohnert, Florian Dick,      tation. Nature Methods, 18(2):203–211, 2021. 6

Joost van Herwaarden, Christos Karkos, Mark Koelemay,[10] Margo O Kaller and Jason An. Contrast agent toxicity. 2019.

Tilo K¨olbel, Ian Loftus, Kevin Mani, Germano Melissano,    1

Janet Powell, Zolt´an Szeberin, Esvs Guidelines Commit-

[11] Alex Kendall, Yarin Gal, and Roberto Cipolla.  Multi-task                                                                                        tee, Gert J de Borst, Nabil Chakfe, Sebastian Debus, Rob

learning using uncertainty to weigh losses for scene geome-                                                                               Hinchliffe, Stavros Kakkos, Igor Koncar, Philippe Kolh,

try and semantics, 2018. 4                                                                            Jes S Lindholt, Melina de Vega, Frank Vermassen, Document

[12] Ilya Loshchilov and Frank Hutter. Decoupled weight decay           Reviewers, Martin Bj¨orck, Stephen Cheng, Ronald Dal-

regularization, 2019. 6                                        man, Lazar Davidovic, Konstantinos Donas, Jonothan Earn-

[13] Andreas Lugmayr, Martin Danelljan, Andres Romero, Fisher           shaw, Hans-Henning Eckstein, Jonathan Golledge, Stephan

Yu, Radu Timofte, and Luc Van Gool. Repaint: Inpainting          Haulon, Tara Mastracci, Ross Naylor, Jean-Baptiste Ricco,

using denoising diffusion probabilistic models, 2022. 4              and Hence Verhagen. Editor’s choice - european society for

[14] Jun Ma, Yuting He, Feifei Li, Lin Han, Chenyu You, and           vascular surgery (ESVS) 2019 clinical practice guidelines on

Bo Wang.  Segment anything in medical images.  Nature            the management of abdominal aorto-iliac artery aneurysms.

Communications, 15(1):654, 2024. 6                            Eur J Vasc Endovasc Surg, 57(1):8–93, 2018. 8

8250[25] Zachary L Whaley, Ismail Cassimjee, Zdenek Novak, David

Rowland, Pierfrancesco Lapolla, Anirudh Chandrashekar,

Benjamin J Pearce, Adam W Beck, Ashok Handa, Regent

Lee, Oxford Abdominal Aortic Aneurysm Study, and Ox-

ford Regional Vascular Services. The spatial morphology of

intraluminal thrombus inﬂuences type II endoleak after en-

dovascular repair of abdominal aortic aneurysms. Ann Vasc

Surg, 66:77–84, 2019. 3, 5, 8

[26] Huiqiao Xie, Yang Lei, Tonghe Wang, Pretesh Patel, Wal-

ter J Curran, Tian Liu, Xiangyang Tang, and Xiaofeng Yang.

Generation of contrast-enhanced  ct with residual cycle-

consistent generative adversarial network (res-cyclegan). In

Medical Imaging 2021: Physics of Medical Imaging, pages

1042–1048. SPIE, 2021. 1

[27] Guang Yang, Jingkun Chen, Xicheng Sheng, Shan Yang,

Xiahai Zhuang, Betty Raman, Lei Li, and Vicente Grau.

Contrast-free myocardial scar segmentation in cine mri using

motion and texture fusion. In 2025 IEEE 22nd International

Symposium on Biomedical Imaging (ISBI), pages 1–5, 2025.

1

[28] Yuqi Yang, Peng-Tao Jiang, Qibin Hou, Hao Zhang, Jinwei

Chen, and Bo Li. Multi-task dense predictions via unleash-

ing the power of diffusion. In The Thirteenth International

Conference on Learning Representations. 3

[29] Hanrong Ye and Dan Xu.  Diffusionmtl: Learning multi-

task denoising diffusion model from partially annotated data,

3

[30] Hantao Zhang, Yuhe Liu, Jiancheng Yang, Shouhong Wan,

Xinyuan Wang, Wei Peng, and Pascal Fua. Lefusion: Con-

trollable pathology synthesis via lesion-focused diffusion

models, 2024. 4

[31] Hantao Zhang, Yuhe Liu, Jiancheng Yang, Weidong Guo,

Xinyuan Wang, and Pascal Fua. Diffatlas: Genai-fying atlas

segmentation via image-mask diffusion, 2025. 3

[32] Richard Zhang, Phillip Isola, Alexei A. Efros, Eli Shecht-

man, and Oliver Wang. The unreasonable effectiveness of

deep features as a perceptual metric, 2018. 4, 6

[33] Jun-Yan Zhu, Taesung Park, Phillip Isola, and Alexei A.

Efros.  Unpaired image-to-image translation using cycle-

consistent adversarial networks, 2020. 6

[34] Qikui Zhu, Yanqing Wang, Shaoming Zhu, and Bo Du. Par-

tial consistent adversarial uniﬁed framework for unsuper-

vised non-contrast ct cross-domain adaptation and segmen-

tation. Pattern Recognition, 165:111638, 2025. 2

Acknowledgement

We acknowledge the contribution by the OxAAA Study

Investigators (in particular Ashok Handa, Pierfrancesco

Lapolla and Anirudh Chandrashekar) and the support by the

Thames Valley Vascular Services, Oxfordshire, UK.

8251

[目录 - 请在 Word 中按 Ctrl+A → F9 刷新]